Thesis for Master of Engineering

Enhancing the efficiency of animal-alternative in-silico drug cardiotoxicity prediction through CUDA-based parallel processing

… 2024

Graduate School Kumoh National Institute of Technology

Department of IT Convergence Engineering

Iga Narendra Pramawijaya

Thesis for Master of Engineering

Enhancing the efficiency of animal-alternative in-silico drug cardiotoxicity prediction through CUDA-based parallel processing

…. 2024

Graduate School Kumoh National Institute of Technology

Department of IT Convergence Engineering

Iga Narendra Pramawijaya

Enhancing the efficiency of animal-alternative in-silico drug cardiotoxicity prediction through CUDA-based parallel processing

Supervisor Ki Moo Lim

This Thesis Presented for the Master of Engineering

… 2024

Graduate School Kumoh National Institute of Technology

Department of IT Convergence Engineering

Iga Narendra Pramawijaya

Approval of the Thesis for the Master of Engineering Submitted by Ali Ikhsanul Qauli

December 2020

Chair of Committee Kwang Soup Song (seal)

Committee Ki Moo Lim (seal)

Committee Hojong Choi (seal)

Graduate School Kumoh National Institute of Technology

Enhancing the efficiency of animal-alternative in-silico drug cardiotoxicity prediction through CUDA-based parallel processing

Iga Narendra Pramawijaya

Department of IT Convergence Engineering, Graduate School Kumoh National Institute of Technology

Abstract

Introduction: The comprehensive in vitro proarrhythmia assay (CiPA) has opened broad opportunities to incorporate in silico experiment as an integral part of drug assessment. Following that, the initial assessment of the Hill coefficient and IC50, which are two important parameters describing drug cellular dynamics, can be included in the in silico experiment as well. The initial assessment consists of Hill fitting and parameters bootstrapping. In this work, I propose a new approach to deploy simple Monte Carlo (MC) simulation with Gaussian scatter incorporated into the calculation of bootstrap samples.

Method: The experimental dose-response data is processed into Hill fitting with the least square method (LSM) to obtain best-fit parameters. These fitted parameters combined with Gaussian scatter of blocking data will act as a data generator for MC simulation that will result in bootstrap samples of Hill coefficient and IC50. Finally, several aspects of those bootstrap samples are assessed such as the 95% confidence interval, the parameter distribution (in the form of the histogram), and finally sensitivity analysis by predicting blocking response from various drug concentrations.

Results: It is found that for insufficient dose-response data, both the proposed MC algorithm and the existing Markov chain Monte Carlo (MCMC) method can jump to the quite high value of uncertainties faster especially with observed mean blocking percentage below 60%. However, both algorithms can yield a similar maximum uncertainty profile except within a region of 15-25% minimum observed mean blocking where the MC method generated higher uncertainty than MCMC. In addition, I was able to show that for sufficient observed experimental data where allowed drug dosage is within this range, the proposed MC method match quite well with existing MCMC in term of distribution of fitted parameters.

Conclusion: The proposed MC method for bootstrapping Hill coefficient and IC50 is quite simpler to implement than the existing MCMC method as it adds only Gaussian sampling into the main Hill fitting algorithm. In addition to its simplicity, the proposed MC method has comparable results with the existing MCMC method in terms of fitted parameter distribution and uncertainty quantification.

CUDA기반 병렬처리를 통한 동물대체 인실리코 약물 심독성 예측 효율성 증대

Iga Narendra Pramawijaya

금오공과대학교 대학원 IT융복합공학과

요 약

소개 : 종합 체외 부정맥 분석(CiPA)은 약물 평가의 필수 부분으로 In silico 컴퓨터 시뮬레이션을 통합할 수 있는 광범위한 기회를 열었다. 그에 따라 약물에 의한 세포 역학을 설명하는 두 가지 중요한 매개 변수인 Hill계수와 IC50의 초기 평가는 In silico 실험에 포함될 수 있다. 이 초기 평가는 Hill 피팅과 매개 변수의 부트스트랩과정으로 구성된다. 본 연구에서는 부트스트랩 샘플의 계산을 위해 가우스 산포가 포함된 몬테카를로(MC) 시뮬레이션을 사용하는 새로운 접근방식을 제안하였다.

방법 : 최소제곱법(LSM)을 사용한 힐 피팅을 통해 실험 복용량-반응 데이터로부터 최적의 매개변수를 생성하였다. 약물에 의한 차단 데이터의 가우스 산포와 결합된 피팅 된 매개변수는 MC 시뮬레이션을 위한 데이터 생성기로 사용되어 Hill계수와 IC50의 부트스트랩 샘플을 생성한다. 부트스트랩 된 샘플들은 95%의 신뢰구간에 분포하는 정도, 매개 변수의 분포(히스토그램의 형태) 및 다양한 약물 농도에 따른 차단 반응의 예측을 통한 민감도 분석을 통해 평가되었다.

결과 : 복용량-반응 데이터가 충분하지 않은 경우 MC 알고리즘과 Markov Chain Monte Carlo (MCMC) 알고리즘 모두에서 매우 높은 불확실성 수치를 생성하였으며, 이는 특히 측정된 평균 차단 비율이 60% 미만인 경우에서 두드러지게 관찰되었다. 하지만, 두 알고리즘 모두 유사한 최대 불확실성 프로파일을 생성할 수 있었으며 최소 측정된 평균 차단 정도가 15~25%인 영역에서는 MC 방법이 MCMC 방법보다 더 높은 불확실성을 생성하였다. 또한, 허용된 약물 복용량이 이 범위 내에서 측정된 실험 데이터가 충분한 경우 연구에서 제안한 MC 방법이 피팅 된 매개 변수의 분포 측면에서 기존의 MCMC 방법과 매우 잘 일치 하는 것을 보여주었다.

결론 : Hill 계수와 IC50을 부트스트랩하기 위해 연구에서 제안한 MC 방법은 주 Hill 피팅 알고리즘에 가우시안 샘플링 과정이 추가되기 때문에 기존의 MCMC 방법에 비해 구현이 쉽다. 이러한 단순성 외에도 제안된 MC방법은 적합 매개 변수의 분포 및 불확실성의 정량화 측면에서 MCMC 방법과 유사한 결과를 낼 수 있었다.

Contents

[List of Figures]	ii

[List of Tables]	iii

[Glossary]	iv

[Acknowledgement]	v

Chapter 1. Introduction	1

1.1 Hill equation and quantitative pharmacology	1

1.1.1 Hill-Langmuir equation	1

1.1.2 Hill equation for dose-response curves	2

1.1.3 The operational model of agonist action	3

1.2 CiPA initiative	5

1.3 Previous Study	6

1.4 Objectives	8

Chapter 2. Methodologies	11

2.1 Fitting dose-response curves	12

2.1.1 Linear LSM	13

2.1.2 Nonlinear LSM	14

2.2 Monte Carlo simulation for generating bootstrap and uncertainties	15

2.3 Comparison between proposed Hill fitting with LSM combined with MC simulation and existing MCMC method	16

Chapter 3. Results and Discussion	18

3.1 Best-fit parameters and their confidence intervals	18

3.2 Uncertainty of blocking response	23

Chapter 4. Conclusion and Limitation	30

[References]	31

Appendix	33

A.	Code for Data processing	33

B.	Code for linear and nonlinear LSM	35

C.	Code for Monte Carlo simulation	38

D.	Complete code for Hill fitting with LSM and Monte Carlo simulation	39

# [List of Figures]

[Figure 1. 1] Linear (left) and logarithmic (right) plotting of the Hill equation. It can be observed on the right panel that the variation of Hill slope n affect the amount of effect for some particular agonist concentration.	3

[Figure 1. 2] The dose-response curve of operational model of agonist action. Note that Effectmax is the maximum possible response of the system while Emax is the one we observe from experiment.	5

[Figure 1. 3] The uncertainty predictions from MCMC calculation adapted from previous study done by Chang et al [15]. Please note that the vertical dashed lines represent the 60% observed mean blocking in experiment.	8

[Figure 1. 4] Overview of proposed computational drug assessment and classification.	9

[Figure 2. 1] The process diagram for obtaining the bootstrap with LSM and Monte Carlo simulation	12

[Figure 2. 2] An example of averaged data for linear LSM fitting.	14

[Figure 3. 1] Comparison of the results from scheme 1 (left) and scheme 2 (right) on Dofetilide hERG channel	22

[Figure 3. 2] Comparison of the results from scheme 1 (left) and scheme 2 (right) on Dofetilide INa channel	23

[Figure 3. 3] The comparison of Monte Carlo simulation of scheme 1 (left) and scheme 2 (right). Note that the gray shaded area is the result from MCMC as reference. Please note that q25-q75 means the blocking response of 2.5 to 97.5 quantiles of the marginal distribution, while q05-q95 for 5 to 95 quantiles.	24

[Figure 3. 4] Uncertainties of blocking response from scheme 1 as a function of observed mean blocking percentage at highest experimental drug concentration (Chigh). Please note that the dashed vertical line means 60% of observed mean blocking at Chigh.	25

[Figure 3. 5] Uncertainties of blocking response from scheme 2 as a function of observed mean blocking percentage at highest experimental drug concentration (Chigh). The vertical dashed line represents the same meaning as in [Figure 3. 4].	26

[Figure 3. 6] The maximum uncertainty as a function of minimum observed average blocking response. Left panel from scheme 1 and right one from scheme 2. Please note that the dashed line represents the result from MCMC.	27

[Figure 3. 7] The blocking response distribution at drug concentration of Chigh from scheme 1 (left) and scheme 2 (right). y axis is the relative frequency and x axis is the blocking percentage. Please note that the middle and lower panels have secondary vertical axis for the results of MCMC.	28

# [List of Tables]

[Table 2. 1] 12 CiPA training drugs and its TdP risk classification	11

[Table 3. 1] Hill equation parameters and confidence intervals (square bracket) for the 12 CiPA training drugs with scheme 1 approach from [Figure 2. 1]	18

[Table 3. 2] Hill equation parameters and confidence intervals (square bracket) for the 12 CiPA training drugs with scheme 2 approach from [Figure 2. 1]	19

# [Glossary]

| TdP | Torsade de pointes |
| --- | --- |
| CiPA | Comprehensive in vitro Proarrhythmia Assay |
| ECG | Electrocardiogram |
| UQ | Uncertainty quantification |
| MC | Monte Carlo |
| MCMC | Markov chain Monte Carlo |
| DRAM | Delaying rejection adaptive Metropolis |
| AARJ | Adaptive automatic reversible jump |
| LSM | Least square method |
| LM | Lavenberg-Marquardt |
| GSL | GNU Scientific library |
| N/A | Not applicable |
| hERG | Human Ether-à-go-go-Related Gene |
| ICaL | L-type calcium current |
| IK1 | Inward rectifier potassium current |
| IKs | Slow delayed rectifier potassium current |
| Ito | Transient outward current |
| INaL | Late sodium current |
| INa | Sodium current |

# [Acknowledgement]

In the name of God, The Most Gracious and The Most Merciful. Alhamdulillah, all praises to God. This thesis would not have been completed without God’s will and guidance. Also, may blessing and peace always be upon Prophet Muhammad, his family, his companion, and his followers all over the world.

First of all, I would like to express my sincere gratitude to my supervisor Professor Ki Moo Lim. Thank you for allowing me to be part of the research member of the Computational Medicine Lab (CML). Your valuable guides and support really help me excel in many aspects I am good at during my study and research in CML. Also, I would like to thank the referee Professor Kwang Soup Song, and Professor Ho Jong Choi for their time, support, and constructive comments.

I would also like to extend my gratitude to all lab mates that I have ever met and collaborated with; Aulia, Getu, Abebe, 다운,하영, Marcell,준효, and 에담. Thank you for all support and advice that had been given to me. I do not think I can survive during my study and research without your help, guys.

For all Indonesian students that I have ever met and collaborated with during my study in Korea, especially those who stayed in Shinpyeong; Aulia, Rahma, Salwa, Naufal, other students from KIT, also from Indonesian Students Association in South Korea (Perpika); Anafi, CQ, and other students from various campuses and cities in Korea that I cannot include here one by one, thank you to be important parts of my life. You really help me excel to become a better human being as a whole. Take care and stay healthy, guys.

Lastly, I wish to express my gratitude to my family, my dad, mom, brother, and sisters. They have always unconditionally supported me to be as best as I can be. Thank you for all that support. May The Almighty God grants you the best life in this small, temporary world and in the eternal afterlife.

# Introduction

Cardiovascular diseases are the leading global causes of death, which emphasizes the importance of effective methods for drug discovery. Traditionally, drug cardiotoxicity prediction is achieved using animal testing, which is controversial and has several drawbacks. Modern in-silico or computer-based methods for drug cardiotoxicity prediction show promising results as an animal-alternative alternative. Nevertheless, some of them are computationally inefficient due to large amount of sample it needs to compute.

## Hill equation and quantitative pharmacology

The scientific paper published by Archibald Vivian Hill, British physiologist, and pharmacologist (1886-1977), in 1910 is known as the first explicit explanation of the Hill equation [1]. The paper describes the interaction of hemoglobin with oxygen and Hill’s formulation is considered as a good approximation for every specific and saturable binding [2]. A few years after Hill’s publication, Irving Langmuir independently published his paper in relation to the adsorption of the gases on the plain surface [3]. This formal relationship between binding substances later was referred to as Hill, Langmuir-Hill, and Hill-Langmuir equation, respectively. Recently, the term “Hill equation” has been defined for the relationship between concentration and effect while the “Hill-Langmuir equation” is for quantifying the ligand-receptor interaction [4].

### Hill-Langmuir equation

The Hill-Langmuir equation can be derived from the mass-action law for equilibrium [2] as follow:

|  | nL+Rk1k2LnR | (Eq. 1.1.1) |
| --- | --- | --- |
|  | LnR=R0⋅LnLn+Kd=R0⋅LnLn+KAn | (Eq. 1.1.2) |

where L is the ligand that presents in variable concentration, R is the amount of receptor, LnR is the concentration of ligand-receptor complex, R0 is the total receptor concentration, L is the free ligand concentration, k1 and k2 are the rate constants of association and dissociation, respectively, Kd=k2/k1 is the equilibrium dissociation constant of the ligand-receptor complex, KA is the ligand concentration where half of the receptors are ligand-bound, and n is commonly known as the Hill coefficient or Hill slope factor.

Note that there are several conditions for the (Eq. 1.1.2) to properly work. Firstly, the binding reaction has to reach equilibrium to ensure association and dissociation do not affect the result [5]. Moreover, the total ligand concentration is assumed not to differ much from the free ligand concentration (in the biological system, it is common that total ligand concentration significantly more populous than receptor concentration). The reaction described in (Eq. 1.1.1) also requires that all binding sites of a receptor should be occupied at once. Hence, only the free L, the free R, and the fully bound LnR forms are permitted.

### Hill equation for dose-response curves

The classical Hill equation for phenomenologically describing the relation between the agonist concentration and the effect has the form as follow:

|  | E=Emax⋅cncn+EC50n | (Eq. 1.1.3) |
| --- | --- | --- |

where c is the agonist concentration, E is the effect yielded by agonist at c, Emax is the maximum effect (upper asymptote) that can be generated by the given agonist in the system, EC50 (effective concentration) is the agonist concentration that evokes half of the maximum effect, n is the Hill slope of Hill coefficient. Note that this equation implicitly assumes that the lower asymptote of the effect is zero. If the system has a basal activity at c=0 and the effect is E≠0, one needs to include the lower baseline effect into the equation.

### The operational model of agonist action

The Hill equation for fitting dose-response curve in (Eq. 1.1.3) can give us information about EC50 and Hill slope factor n of the pharmacological agent. However, the EC50 and n alone cannot tell us everything about the agonist because the EC50 is also determined by two properties of agonist[6]:

- Affinity, a quantity that tells us how well the agonist binds to a receptor.

- Efficacy, a quantity that describes how well the agonist causes a response once bound.

[Figure 1. 1] Linear (left) and logarithmic (right) plotting of the Hill equation. It can be observed on the right panel that the variation of Hill slope n affect the amount of response as a function of agonist concentration.

To obtain a more comprehensive understanding of the action of agonist and partial agonist (agonist that partially activates receptor), Black and Leff proposed the operational model of agonism [7]. This model combines the occupancy equation with the transducer function that mathematically can be expressed as follow:

|  | Effect=EffectmaxτncnKA+cn+τncn | (Eq. 1.1.4) |
| --- | --- | --- |

where c is the agonist concentration, τ is the “transducer constant”, KA is the agonist-receptor equilibrium dissociation constant, and n is the slope parameter. In comparison to the Hill equation, one can calculate the Emax and EC50 as follow:

As in [Figure 1. 2], the Effect will reach Emax at very high agonist concentration. Mathematically one can write as

|  | Emax=limc→∞EffectmaxτncnKA+cn+τncn | (Eq. 1.1.5) |
| --- | --- | --- |

that with some algebra one can obtain:

|  | Emax=Effectmaxτnlimc→∞1τn+KA+ccn | (Eq. 1.1.6) |
| --- | --- | --- |
|  | Emax=Effectmaxτnτn+1 | (Eq. 1.1.7) |

For EC50, again from [Figure 1. 2], the Effect will reach Emax/2 when c=EC50, one can combine (Eq. 1.1.4) and (Eq. 1.1.7) to obtain the following expressions:

|  | 12τn+1=1τn+KA+EC50EC50n | (Eq. 1.1.8) |
| --- | --- | --- |
|  | EC50=KA2+τn1/n -1 | (Eq. 1.1.9) |

Please note that n is not the Hill slope even though those values can be very close for full agonist.

[Figure 1. 2] The dose-response curve of the operational model of agonist action. Note that Effectmax is the maximum possible response of the system while Emax is the one we observe from the experiment.

Suppose we compare the Hill equation and the operational model of agonism. On one hand, the Hill equation can yield some parameters that may lack physicochemical meaning where the parameters can only geometrically characterize the dose-respond curves [8, 9]. On the other hand, the mechanistic models may allow for a deeper understanding of the binding process with some drawbacks that it requires detailed prior knowledge about particular mechanisms of the signal-transduction [5] as well as too many additional fitting parameters may yield big standard errors [6, 10]. From that comparison, we could understand that under some circumstances, it is feasible to use the empirical model of the Hill equation that requires minimum previous knowledge of the agonist. Thus, the first step to screen the biological activity of pharmacological agents is to make the E/c curves and fit them with the Hill equation [5].

## CiPA initiative

Torsade de pointes (TdP) is a heart abnormality that can lead to sudden heart death. Nowadays, drug-induced TdP has been responsible for the rejection of many drugs from the market [11] and becomes a major concern for the pharmaceutical industry as well as global regulatory agencies. Related to the previously described models of agonist action, the drug can induce ionic current blocking within the ventricular tissue. Therefore, assessing the safety of drug-induced proarrhythmic risk is becoming an important concern as well.

The present cardiac safety paradigm of ICH S7B non-clinical guidance and E14 clinical guidance has provided basis regulation for detection of the nonclinical focus on the blocking of the repolarizing potassium ionic current, represented as IKr, through hERG channels and clinical focus on a surrogate marker of proarrhythmia namely QTc prolongation. Nevertheless, the current paradigm does not examine directly the ventricular arrhythmia like TdP. Sager et al [12] described some major limitations within the present approach including the repolarization that is not sufficiently predicted by the block of IKr criteria alone, the prolongation of QTc is rather sensitive but not specific for predicting ventricular proarrhythmia risk, and lastly, some drugs are not proarrhythmic risk but able to block IKr

A new safety paradigm entitled “Comprehensive in vitro Proarrhythmia Assay” (CiPA) has been proposed to assess overall proarrhythmia risk. As described by Vicente et al [13], CiPA consist of four main components: in vitro assessment of drug-induced effects on multiple ionic currents; in silico computer modeling; in vitro drug effects on human induced pluripotent stem cell-derived ventricular cardiomyocytes (hiPSC-CMs); and the assessment made during phase 1 clinical trials. Furthermore, Strauss et al [14] explained that the in vitro experimental data will focus on three dominant plateau currents of hERG, late sodium, and calcium. Also, within the in silico computation of CiPA, the drug-induced TdP risk will be predicted with qNet as a Torsade Metric Score that classifies the drugs into low, intermediate, and high-risk categories. Moreover, the unexpected effect on humans will be determined by electrocardiograms (ECG) biomarker such as the heart rate-corrected J-Tpeak interval (J-Tpeakc), the time between the end of the QRS interval and the peak of T-wave.

## Previous Study

Parallelisation in computational biology is not an entirely new concept. The Cells in Silico (CiS) framework presented by Berghoff et al. (2020) [cite] offers a tool for simulating the growth and development of biological tissues. The modular and parallel design of CiS allows for flexible configuration of different model assumptions, making it applicable to a wide range of research questions. As demonstrated by the example of a 10003 voxel-sized cancerous tissue simulation at sub-cellular resolution, CiS can be used to explore complex biological processes at a high level of detail.

Utilisation of GPU in biological cell computing has been explored in previous researches. One of them is from Miguel, et al [cite] in 2020. Miguel, et al. explored an adaptive parallel simulator to solve performance loss in massive parallel membrane computing devices known as membrane systems or P systems. The paper demonstrates the effectiveness of this approach by extending an existing simulator for Population Dynamics P systems. Experimental results show that this adaptive simulation can significantly improve performance, up to 2.5x on both GPUs and multicore processors.

Related to drug toxicity and discovery, other researchers tried to approach and optimise drug development process using parallel computing approach as well. Previously, McIntosh-Smith et, al. developed a in-silico drug screening method on multiple core processors. McIntosh-Smith et, al. developed BUDE (Bristol University Docking Engine), a drug discovery tool, simulating molecular docking. To speed up calculations on powerful processors with multiple cores, BUDE has been adapted to work with OpenCL, a common language for parallel programming [cite]. As a result, McIntosh-Smith et, al. achieved of 46% at peak, or 1.43 TFLOP/s on a single Nvidia GTX 680.

Barth et, al. developed a parallelisation on biochemical simulation of metabolic pathways in their high level computational simulation. This method allows Barth et, al. to run simulations with more complex models, featuring a greater number of chemicals and reactions. Hence, Barth et, al. can achieve more realistic, lifelike outcomes while using less computing time [cite].

[Figure 1. 3] The uncertainty predictions from MCMC calculation adapted from a previous study done by Chang et al [15]. Please note that the vertical dashed lines represent the 60% observed mean blocking in the experiment.

## Objectives

The research group of Computational Medicine Laboratory (CML) in Kumoh National Institute of Technology (KIT) has proposed computational simulation for drug assessment as follow:

[Figure 1. 4] Overview of proposed computational drug assessment and classification.

From [Figure 1. 1], drug-response data from the experiment is processed by several computational steps: Hill fitting and Monte Carlo simulation to obtain bootstrap samples; in silico simulation of ventricular cardiomyocyte action potential (AP) to obtain a set of biomarker information like a net charge of ionic currents (qNet); drug classification with machine learning by using previous biomarker information.

This study aims to provide the first step of computational drug assessment that consists of Hill fitting and parameters bootstrapping. To achieve accurately fitted parameters and reliable predictions, the author incorporated a nonlinear fitting procedure and Monte Carlo simulation with Gaussian scatter into the algorithm to mimic the actual spread of dose-response experimental data. The proposed algorithm is also compared with the existing Markov chain Monte Carlo (MCMC) method as shown by [18] for obtaining bootstrap parameters. Furthermore, the author examines various aspects of the proposed algorithm in comparison with previous work that used MCMC as shown by another research group [15] such as the confidence intervals for the fitted parameters and the uncertainty of blocking response.

# Methodologies

In this work, the author examines the 12 training drugs as the one in [15] that can be seen in the table below:

[Table 2. 1] 12 CiPA training drugs and its TdP risk classification

| Drug | CiPA TdP risk |
| --- | --- |
| Dofetilide | High |
| Bepridil | High |
| Sotalol | High |
| Quinidine | High |
| Cisapride | Intermediate |
| Terfenadine | Intermediate |
| Ondansetron | Intermediate |
| Chlorpromazine | Intermediate |
| Verapamil | Low |
| Ranolazine | Low |
| Mexiletine | Low |
| Diltiazem | Low |

Meanwhile, the process diagram for obtaining the bootstrap samples of logIC50 and h with the least square method (LSM) and Monte Carlo (MC) simulation is as the following [Figure 2. 1]:

[Figure 2. 1] The process diagram for obtaining the bootstrap with LSM and Monte Carlo simulation

The overall procedure starts from the fitting process that contains linear and nonlinear fitting. The results of the fitting process will be the best-fit parameters that will be used in the next process of bootstrapping by using MC simulation. There are two available schemes for MC bootstrapping; scheme 1 is using best-fit parameters to generate initial data while scheme 2 is incorporating the actual experimental data. The final results from bootstrapping are the 2,000 bootstrap samples that will be used for the next stage of in silico AP simulation afterward.

## Fitting dose-response curves

The drug’s dose-response curve may have a similar sigmoid form as on the right panel of [Figure 1. 1] for most of the drug. As long as the binding steps follow the law of mass action, the curve will follow the hyperbolic or sigmoid shape [6]. It is also shown by the mechanistic approach by Black and Leff [7] that no matter how many steps intervene during the binding, as long as it follows the law of mass action, the response will follow the sigmoid form as in [Figure 1. 2]. Thus, for fitting the dose-response curve of the drug, the formula used to fit is the standard Hill equation in (Eq. 1.1.3) with some modifications: E to be replaced by Block because the effect will be the blocking of the ionic channel; n to be replaced by h to resemble Hill slope or coefficient; and EC50 to be replaced by IC50 that stands for inhibitory concentration. Also, instead of using drug concentration c in the x-axis, logc is used as the x-axis independent variable which is typical in dose-response curve fitting.

In this work, the fitting algorithm to be used is the nonlinear least square method. However, there are some prior steps before employing the nonlinear fitting to obtain final fitting results that will be explained in the next sub-sections.

### Linear LSM

As depicted in [Figure 2. 1], the initialization section is finalized by nonlinear LSM that needs appropriate initial input values (logIC50 and h) in order to obtain good results. For that purpose, linear LSM is used to the experimental data to obtain initial logIC50 and h. The Hill equation (Eq. 1.1.3) needs to be arranged as follow:

|  | y=100chch+IC50h | (Eq. 2.1.1) |
| --- | --- | --- |

where y is the blocking effect (in percentage) of the drug.

By taking the logarithmic value of both sides and assigning new X and Y as the following expression:

|  | Y=log11-y100-1 | (Eq. 2.1.2) |
| --- | --- | --- |
|  | X=logc | (Eq. 2.1.3) |

thus one can obtain the linear formula as follow:

|  | Y=A+BX | (Eq. 2.1.4) |
| --- | --- | --- |

where A= -hlogIC50 and B=h.

The data for linear LSM fitting has to be processed as well. In this case, we use the “averaged” data to be fitted in the linear function of (Eq. 2.1.4). The blocking percentage within the same dose of the drug will be averaged. An example of that is as the following figure:

[Figure 2. 2] An example of averaged data for linear LSM fitting.

The linear LSM fitting algorithm is minimizing the sum of the square error function χ2 as follow:

|  | χ2=iYyi-A+BXxi2 | (Eq. 2.1.5) |
| --- | --- | --- |

where Y is the linear model as in (Eq. 2.1.2), xi,yi is dose-response data. The results from linear regression are the best-fit parameters A and B from which the initial value of logIC50=-A/B and h=B can be obtained. These results will serve as an initial value for nonlinear LSM fitting. The implementation of linear LSM is done by using the ALGLIB C++ library.

### Nonlinear LSM

The nonlinear LSM applied in this work follows the Lavenberg-Marquardt (LM) method as proposed by [20, 21]. The sum of square error Φ from residual function fi to minimize is expressed as follow:

|  | Φ=ifi2 | (Eq. 2.1.6) |
| --- | --- | --- |
|  | fi=blocki-yi | (Eq. 2.1.7) |
|  | blocki=1001-11+xi10logIC50h | (Eq. 2.1.8) |

where xi is the drug concentration and yi is the blocking percentage from the experiment. The implementation of the LM algorithm is done by using the ALGLIB C++ library. The library code requires a blocking function as in (Eq. 2.1.1).

Furthermore, in this work, the author applied bound constraints to the fitted parameters as follow:

|  | -10≤logIC50≤10 | (Eq. 2.1.9) |
| --- | --- | --- |
|  | 0≤h≤10 | (Eq. 2.1.10) |

The results from nonlinear LSM are the best-fit of logIC50best and hbest which will be used for further process in MC simulation as in section 2.2. The implementation of nonlinear LSM is done by using the ALGLIB C++ library.

## Monte Carlo simulation for generating bootstrap and uncertainties

In this work, I followed the MC simulation for generating bootstrap samples as proposed by [6]. From [Figure 2. 1], there are two available schemes of MC simulation. The first scheme uses the best-fit parameters obtained in the initialization section to generate dose-response data for MC simulation, while the second one uses directly the actual experimental data. We can see more clearly how MC simulation works for both scheme as follow:

- First, generate the dose-response data for MC simulation with Gaussian scatter on each data on y axis. For scheme 1, the generated dose-response data can be expressed as block1i=blocki+N0,SD, while for scheme 2 it is expressed as block2i=yi+N0,SD. Please note that the expression of N0,SD means Gaussian sampling with zero mean and standard deviation of SD. The standard deviation of SD described as

|  | SD=iblocki-yi2n-p | (Eq. 2.2.1) |
| --- | --- | --- |

where n is the number of data, p is the number of parameters to fit (in our case it is 2), and blocki is the model that follows (Eq. 2.1.8) with parameters logIC50best.and hbest from initialization section.

- Run linear and nonlinear LSM fitting to obtain new fit parameters of logIC50 and h.

- Repeat steps (1) and (2) as many samples as we need. In this work, I run 2,000 times, thus I would obtain 2,000 bootstrap samples of logIC50 and h.

Within the implementation in code, I embedded the ALGLIB C++ library for the linear and nonlinear LSM in step (2) above. Meanwhile, for Gaussian random sampling of the data error in step (1) above, I used the GNU Scientific Library (GSL).

The bootstrap samples generated from the Monte Carlo simulation can then be used for further analysis such as finding confidence intervals for each parameter. For example, to find a 95% confidence interval of the parameter, one needs to find 2.5 and 97.5 percentiles from the bootstrap samples. The 2.5 percentile will be the lower bound, and the 97.5 percentile will be the upper bound of the 95% confidence interval.

## Comparison between proposed Hill fitting with LSM combined with MC simulation and existing MCMC method

To gain confidence in deploying the proposed methodologies in sections 2.1.1 and 2.1.2, I compared the results from the proposed method with the existing MCMC algorithm. Some aspects that can be compared are as follows:

- The best-fit parameters and its 95% confidence interval.

- The distribution of fit parameters in the form of histogram plots.

- The uncertainty of blocking response in the form of 95% confidence interval of blocking percentage as a function of logarithmic of drug concentration.

These aspects are deployed for all 12 CiPA training drugs as in [Table 2. 1]. The results from MCMC and experimental dose-response data are gathered from https://github.com/FDA/CiPA.

# Results and Discussion

## Best-fit parameters and their confidence intervals

The outcomes from LSM fitting will be the optimum or best-fit parameters of logIC50 and h. It is found that the best-fit parameters from Hill fitting with LSM match exactly with previous work by [15]. Meanwhile, the 95% confidence interval of the parameters from two schemes shows different results among them that can be seen in [Table 3. 1] and [Table 3. 2]. In each table, the N/A or “not applicable” means the fitted IC50 is not well defined as no detectable blocking appeared during experiment as stated by Li et al [22].

[Table 3. 1] Hill equation parameters and confidence intervals (square bracket) for the 12 CiPA training drugs with scheme 1 approach from [Figure 2. 1]

|  | Dofetilide |  | Cisapride |  |
| --- | --- | --- | --- | --- |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 2.42[1.80,10.0] | 1.16[0.21,1.70] | 6.97[4.69,10.0] | 0.43[0.24,0.70] |
| IK1 | 2.60[1.47,10.0] | 0.77[0.15,1.69] | 4.47[3.33,10.0] | 0.51[0.15,0.81] |
| IKs | N/A | N/A | 7.91[4.63,10.0] | 0.29[0.20,0.60] |
| Ito | 1.27[1.04,1.79] | 0.77[0.47,1.11] | 5.34[3.55,10.0] | 0.24[0.11,0.46] |
| INaL | 5.88[1.81,10.0] | 0.26[0.13,1.05] | N/A | N/A |
| INa | 2.58[1.63,10.0] | 0.90[0.19,1.76] | N/A | N/A |
| hERG | 0.16[0.09,0.23] | 0.63[0.52,0.77] | 1.07[1.02,1.12] | 1.34[1.17,1.55] |
|  | Bepridil |  | Verapamil |  |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 3.45[3.30,3.67] | 0.65[0.47,0.85] | 2.30[2.23,2.38] | 1.10[0.88,1.35] |
| IK1 | N/A | N/A | 8.54[4.55,10.0] | 0.27[0.19,0.80] |
| IKs | 4.46[4.09,5.47] | 0.71[0.39,1.04] | N/A | N/A |
| Ito | 3.93[3.63,10.0] | 3.54[0.26,10.0] | 4.13[3.58,5.83] | 0.82[0.37,1.48] |
| INaL | 3.26[3.20,3.31] | 1.42[1.15,1.70] | 3.85[3.26,10.0] | 1.03[0.15,2.73] |
| INa | 3.47[3.39,3.58] | 1.16[0.84,1.51] | N/A | N/A |
| hERG | 2.17[1.96,2.36] | 0.88[0.61,1.33] | 2.70[2.64,2.75] | 1.10[0.93,1.28] |
|  | Terfenadine |  | Ranolazine |  |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 2.85[2.79,2.90] | 0.66[0.60,0.72] | N/A | N/A |
| IK1 | N/A | N/A | N/A | N/A |
| IKs | 5.60[4.27,10.0] | 0.54[0.21,1.03] | 7.56[5.96,10.0] | 0.52[0.29,0.96] |
| Ito | 5.38[4.30,9.01] | 0.26[0.12,0.39] | N/A | N/A |
| INaL | 4.30[3.83,5.50] | 0.60[0.35,0.89] | 3.90[3.80,3.99] | 0.94[0.76,1.16] |
| INa | 3.68[3.26,5.06] | 1.02[0.39,2.14] | 4.84[4.62,5.67] | 1.42[0.60,2.40] |
| hERG | 1.27[1.08,1.45] | 0.59[0.45,0.75] | 3.81[3.71,3.92] | 0.84[0.66,1.02] |
|  | Sotalol |  | Mexiletine |  |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 6.85[6.68,7.17] | 0.87[0.61,1.14] | 4.58[4.37,5.00] | 1.03[0.67,1.48] |
| IK1 | 6.48[6.41,6.59] | 1.20[0.93,1.53] | N/A | N/A |
| IKs | 6.63[6.51,6.84] | 1.17[0.80,1.58] | N/A | N/A |
| Ito | 7.63[6.95,9.98] | 0.66[0.28,1.29] | N/A | N/A |
| INaL | N/A | N/A | 3.95[3.89,4.03] | 1.41[1.06,1.80] |
| INa | 9.06[7.52,10.0] | 0.51[0.35,1.11] | N/A | N/A |
| hERG | 4.94[4.78,5.08] | 0.94[0.73,1.23] | 4.39[4.20,5.05] | 2.59[1.03,4.94] |
|  | Quinidine |  | Ondansetron |  |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 4.71[4.48,5.04] | 0.59[0.47,0.73] | 4.35[4.23,4.54] | 0.75[0.57,0.95] |
| IK1 | 7.60[4.88,10.0] | 0.35[0.19,0.91] | N/A | N/A |
| IKs | 3.69[3.63,3.78] | 1.36[1.00,1.76] | 5.76[5.29,6.83] | 0.65[0.40,0.93] |
| Ito | 3.54[3.50,3.59] | 1.28[1.09,1.51] | 6.01[5.19,10.0] | 0.99[0.27,1.67] |
| INaL | 3.97[3.91,4,08] | 1.34[1.06,1.63] | 4.28[4.21,4.37] | 1.03[0.82,1.25] |
| INa | 4.09[3.97,4.33] | 1.49[0.99,2.08] | 4.76[4.51,6.35] | 1.02[0.31,1.77] |
| hERG | 2.54[2.40,2.63] | 1.03[0.78,1.33] | 3.17[3.03,3.31] | 0.96[0.71,1.29] |
|  | Diltiazem |  | Chlorpromazine |  |
|  | log10IC50 | h | log10IC50 | h |
| ICaL | 2.05[1.91,2.19] | 0.71[0.55,0.92] | 3.91[3.80,4.04] | 0.84[0.63,1.05] |
| IK1 | N/A | N/A | 3.97[3.84,4.14] | 0.69[0.53,0.85] |
| IKs | N/A | N/A | N/A | N/A |
| Ito | 9.45[6.89,10.0] | 0.17[0.14,0.29] | 7.25[5.55,10.0] | 0.37[0.20,0.68] |
| INaL | 4.34[4.24,4.46] | 0.68[0.56,0.81] | 3.66[3.59,3.73] | 0.94[0.79,1.09] |
| INa | 5.04[4.76,6.07] | 0.70[0.37,0.94] | 3.66[3.58,3.73] | 2.00[1.65,2.46] |
| hERG | 3.82[3.73,3.92] | 0.79[0.67,0.91] | 3.05[2.94,3.16] | 0.89[0.71,1.09] |

[Table 3. 2] Hill equation parameters and confidence intervals (square bracket) for the 12 CiPA training drugs with scheme 2 approach from [Figure 2. 1]

|  | Dofetilide |  | Cisapride |  |
| --- | --- | --- | --- | --- |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 2.42[1.90,10.0] | 1.16[0.21,1.52] | 6.97[5.37,10.0] | 0.43[0.23,0.56] |
| IK1 | 2.60[1.72,10.0] | 0.77[0.14,1.19] | 4.47[3.69,10.0] | 0.51[0.15,0.62] |
| IKs | N/A | N/A | 7.91[5.30,10.0] | 0.29[0.19,0.47] |
| Ito | 1.27[1.06,1.73] | 0.77[0.49,1.03] | 5.34[3.78,10.0] | 0.24[0.11,0.40] |
| INaL | 5.88[1.96,10.0] | 0.26[0.13,0.90] | N/A | N/A |
| INa | 2.58[1.78,10.0] | 0.89[0.19,1.47] | N/A | N/A |
| hERG | 0.16[0.09,0.23] | 0.63[0.52,0.76] | 1.07[1.02,1.12] | 1.34[1.17,1.54] |
|  | Bepridil |  | Verapamil |  |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 3.45[3.31,3.67] | 0.65[0.47,0.81] | 2.30[2.22,2.39] | 1.10[0.88,1.37] |
| IK1 | N/A | N/A | 8.54[4.60,10.0] | 0.27[0.18,0.75] |
| IKs | 4.46[4.20,5.34] | 0.71[0.41,0.88] | N/A | N/A |
| Ito | 3.93[3.63,10.0] | 3.54[0.26,10.0] | 4.13[3.80,5.52] | 0.82[0.40,1.08] |
| INaL | 3.29[3.20,3.31] | 1.42[1.14,1.68] | 3.85[3.27,10.0] | 1.03[0.15,2.70] |
| INa | 3.47[3.39,3.59] | 1.16[0.78,1.63] | N/A | N/A |
| hERG | 2.17[1.97,2.36] | 0.88[0.61,1.29] | 2.70[2.64,2.75] | 1.10[0.90,1.34] |
|  | Terfenadine |  | Ranolazine |  |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 2.85[2.80,2.91] | 0.66[0.60,0.72] | N/A | N/A |
| IK1 | N/A | N/A | N/A | N/A |
| IKs | 5.60[4.60,10.0] | 0.54[0.21,0.83] | 7.56[6.37,10.0] | 0.52[0.29,0.76] |
| Ito | 5.38[3.96,10.0] | 0.26[0.11,0.49] | NA | NA |
| INaL | 4.30[3.91,5.86] | 0.60[0.31,0.79] | 3.90[3.80,4.00] | 0.94[0.76,1.14] |
| INa | 3.68[3.36,5.13] | 1.02[0.38,1.63] | 4.84[4.62,5.68] | 1.42[0.60,2.32] |
| hERG | 1.27[1.08,1.45] | 0.59[0.45,0.81] | 3.81[3.70,3.92] | 0.84[0.68,0.98] |
|  | Sotalol |  | Mexiletine |  |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 6.85[6.68,7.18] | 0.87[0.60,1.16] | 4.58[4.42,4.98] | 1.03[0.68,1.31] |
| IK1 | 6.48[6.42,6.59] | 1.20[0.93,1.45] | N/A | N/A |
| IKs | 6.63[6.51,6.84] | 1.17[0.82,1.52] | N/A | N/A |
| Ito | 7.63[7.29,9.63] | 0.66[0.30,0.82] | N/A | N/A |
| INaL | N/A | N/A | 3.95[3.89,4.03] | 1.41[1.08,1.73] |
| INa | 9.06[7.61,10.0] | 0.51[0.35,0.97] | N/A | N/A |
| hERG | 4.94[4.78,5.08] | 0.94[0.73,1.24] | 4.39[4.22,5.11] | 2.59[0.98,4.47] |
|  | Quinidine |  | Ondansetron |  |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 4.71[4.53,5.02] | 0.59[0.47,0.69] | 4.35[4.23,4.54] | 0.75[0.56,0.92] |
| IK1 | 7.60[4.51,10.0] | 0.35[0.18,1.29] | N/A | N/A |
| IKs | 3.69[3.63,3.78] | 1.36[0.96,1.99] | 5.76[5.45,6.84] | 0.65[0.40,0.78] |
| Ito | 3.54[3.50,3.59] | 1.28[1.07,1.52] | 6.01[5.17,10.0] | 0.99[0.27,1.74] |
| INaL | 3.97[3.91,4.09] | 1.34[1.04,1.58] | 4.28[4.21,4.38] | 1.03[0.80,1.32] |
| INa | 4.09[3.99,4.32] | 1.49[1.02,1.90] | 4.76[4.50,6.17] | 1.02[0.33,1.81] |
| hERG | 2.54[2.41,2.63] | 1.02[0.77,1.39] | 3.17[3.02,3.31] | 0.96[0.72,1.24] |
|  | Diltiazem |  | Chlorpromazine |  |
|  | log10IC50 | H | log10IC50 | h |
| ICaL | 2.05[1.90,2.19] | 0.71[0.55,0.92] | 3.91[3.80,4.06] | 0.84[0.64,1.08] |
| IK1 | N/A | N/A | 3.97[3.84,4.14] | 0.69[0.53,0.82] |
| IKs | N/A | N/A | N/A | N/A |
| Ito | 9.45[7.11,10.0] | 0.17[0.14,0.27] | 7.25[6.11,10.0] | 0.37[0.19,0.50] |
| INaL | 4.34[4.25,4.48] | 0.68[0.55,0.79] | 3.66[3.59,3.73] | 0.94[0.79,1.09] |
| INa | 5.04[4.81,6.01] | 0.70[0.38,0.88] | 3.66[3.58,3.73] | 2.00[1.64,2.45] |
| hERG | 3.82[3.72,3.90] | 0.79[0.66,0.88] | 3.05[2.94,3.15] | 0.89[0.70,1.12] |

Further analysis can be carried out with the plot of the relationship among fitted parameters (logIC50 and h) as well as its histogram as shown in some sample results on [Figure 3. 1] for Dofetilide hERG channel and [Figure 3. 2] for Dofetilide INa channel. For cases with sufficient experimental data like the Dofetilide hERG channel on [Figure 3. 1], both MCMC and the proposed simulation yield similar spread of fitted parameters where there is no strong relation between logIC50 and h for both schemes (see top panels). The histogram plots also show that the posterior distribution of logIC50 and h predicted by MCMC are similar to the MC’s results which resemble Gaussian distribution (see middle and bottom panels). On the other hand, the Dofetilide INa channel on [Figure 3. 2] yields inverse proportionality of two fit parameters (see on top panels). Furthermore, the histogram plots show a completely non-Gaussian shape of parameter distribution. Both MCMC and proposed method result logIC50 mostly grouped near its lower bounds while for h also grouped near its lower bound except that proposed algorithm yield quite high relative frequency on upper bound of h.

Moreover, from the histogram in the middle and bottom panel of [Figure 3. 1] and [Figure 3. 2], one can clearly predict and compare the confidence intervals of fit parameters between the proposed MC algorithm and MCMC. For cases like the Dofetilide hERG channel, where the histogram of MCMC and MC algorithm overlap quite well, the confidence interval of fit parameters would be similar. However, for cases with insufficient drug response like the Dofetilide INa channel where the histogram peak is not well defined and its shape does not resemble Gaussian distribution, the confidence intervals from MCMC and MC algorithm can be completely different and hardly meaningful. The term “insufficient dose-response data” will be discussed in the next subsection.

[Figure 3. 1] Comparison of the results from scheme 1 (left) and scheme 2 (right) on Dofetilide hERG channel

Note that some histogram plots in [Figure 3. 2] (the bottom panels) have a secondary vertical axis on the right side of the panel that shows the results from MCMC. The secondary vertical axis is necessary for some cases because the results from our algorithm grouped massively near either lower or upper bounds of parameters that could yield a relatively huge number of relative frequency compared to MCMC. In the other words, the secondary vertical will not be used where the histogram vertical value of the proposed algorithm is comparable to MCMC. More detailed figures and comparisons for all drugs can be seen in supplementary figures (https://intip.in/NA8X).

[Figure 3. 2] Comparison of the results from scheme 1 (left) and scheme 2 (right) on Dofetilide INa channel

## Uncertainty of blocking response

In the previous subsection, it is obtained that while the results from proposed nonlinear LSM fitting match exactly with the one from the previous study by [15], the confidence intervals can show quite different results compared to MCMC for cases with insufficient dose-response data. The term “insufficient” can be seen from blocking response in sample results of [Figure 3. 3] that maximum blocking response obtained from experiment are very small that some cases have even lower than 20% (Dofetilide ICaL and INa channel).

[Figure 3. 3] The comparison of Monte Carlo simulation of scheme 1 (left) and scheme 2 (right). Note that the gray shaded area is the result of MCMC as a reference. Also note that q25-q75 means the blocking response of 2.5 to 97.5 quantiles of the marginal distribution, while q05-q95 for 5 to 95 quantiles.

In [Figure 3. 3], the bootstrap samples of fit parameters are deployed to obtain a prediction of drug blocking response of the channel. Each bootstrap sample will be used for generating a prediction of blocking response with 100 different drug concentrations. Scheme 1 generally yields a broader range of blocking response compared to scheme 2. However, both schemes can produce a quite similar prediction region only on the Dofetilide hERG channel where at the same time MCMC yields a similar response. This can happen because the sufficient observed blocking responses of the drug, which are about 60-70%, is achieved. Also, from the sample figure on [Figure 3. 3], one can clearly observe that for sufficient experimental data, like the one from the hERG channel, both schemes 1 and 2 generate overlapping regions to the one from MCMC. However, when the blocking response is insufficient, like the one on ICaL and INa channels, the prediction of MCMC yields broader blocking intervals compared to both schemes 1 and 2. This also happens on most of the non-hERG channels. More detailed figures and comparisons for all drugs can be seen in supplementary figures (https://intip.in/NA8X).

[Figure 3. 4] Uncertainties of blocking response from scheme 1 as a function of observed mean blocking percentage at the highest experimental drug concentration (Chigh). Please note that the dashed vertical line means 60% of observed mean blocking at Chigh.

Furthermore, the insufficient observed blocking response can lead to high uncertainty in predicting the drug response especially during extrapolation with bootstrap parameters. In this work, I defined the uncertainty as to the width of 95% confidence interval of blocking the response of the corresponding ionic channel. [Figure 3. 4] and [Figure 3. 5] show that both scheme yield high uncertainty (higher than 16%) when the observed mean blocking is less than 60%, the same as results obtained by a previous study [15] in [Figure 1. 3]. However, while the minimum threshold for observed mean blocking is the same as previous work, the proposed MC algorithm yields a different spread of uncertainty as shown in [Figure 3. 4] and [Figure 3. 5] compared to MCMC in [Figure 1. 3] especially for cases with low observed mean blocking (below 60%). MCMC generates significant jumps in the spread of uncertainties for 2×Chigh to 10×Chigh while both scheme 1 and 2 show a less significant jump in the sense that fewer cases reach high uncertainties with the same drug concentration and observed mean blocking response.

[Figure 3. 5] Uncertainties of blocking response from scheme 2 as a function of observed mean blocking percentage at the highest experimental drug concentration (Chigh). The vertical dashed line represents the same meaning as in [Figure 3. 4].

Further analysis from [Figure 3. 6] can reveal that both MCMC and the proposed MC algorithm show a similar spread of maximum uncertainty when the minimum observed blocking is applied to the blocking response prediction in [Figure 3. 4] and [Figure 3. 5]. One can see that within the range of 15-25% of minimum observed average blocking response, MCMC clearly yields higher uncertainty compared to the proposed method for most drug concentration except 10×Chigh(red lines). This happens because at 10×Chighthe blocking response prediction from MCMC can reach the highest possible blocking response quicker than the proposed algorithm. Therefore, while the MCMC already reaches saturation (the blocking responses are close to 100%), the proposed MC algorithm still in the process to reach it, thus gives higher uncertainty.

[Figure 3. 6] The maximum uncertainty as a function of the minimum observed average blocking response. Left panel from scheme 1 and the right one from scheme 2. Please note that the dashed line represents the result of MCMC.

In addition, from [Figure 3. 6] we can notice that below 60% of the minimum observed average blocking response, the prediction will have uncertainty higher than 16% for both MCMC and proposed MC algorithm. However, for drug concentration at 1×Chigh, the uncertainty is 20% or less for all minimum observed blocking response. This indicates that within the range of experimental data, both MCMC and the proposed MC algorithm can provide a good prediction of the blocking response. In addition, the 60% minimum threshold obtained from [Figure 3. 4], [Figure 3. 5], and [Figure 3. 6] emphasize the results from [Figure 3. 3] that the experimental design needs to be changed if the researchers need more reliable prediction. The change that can be made is at least the minimum observed average blocking response has to be 60%. Thus, from that modification, researchers can expect the uncertainty of blocking responses to be less than 16% for all range of drug concentrations.

[Figure 3. 7] The blocking response distribution at a drug concentration of Chigh from scheme 1 (left) and scheme 2 (right). y axis is the relative frequency and x axis is the blocking percentage. Please note that the middle and lower panels have a secondary vertical axis for the results of MCMC.

One important assumption in the proposed MC simulation is that the scatter of data around the curves follows Gaussian distribution. One can clearly observe in [Figure 3. 7] that for all cases (including the cases with insufficient dose-response data), the blocking response follows the Gaussian distribution. Meanwhile, MCMC generates Gaussian scatter of data only for the Dofetilide hERG channel (top panels) which has sufficient dose-response data. On the other hand, for insufficient cases, as shown in the middle and bottom panels, MCMC yields quite irregular distribution as a relatively huge number of blocking responses are grouped close to its minimum value. This indicates that my assumption for the scattering of data is appropriate whenever sufficient data is achieved and MCMC results emphasize that assumption.

# Conclusion and Limitation

Hill fitting and bootstrapping can play an important role as integral parts of drug assessment in the CiPA project. Providing simple yet reliable parameter fitting and predicting capabilities, I showed that Hill fitting with LSM and Monte Carlo simulation could help researchers during their first step of computational drug assessment. In addition, the proposed MC method generates quite similar results compared to the rather complicated existing MCMC method, both in producing best-fit parameters and predicting the uncertainties. These advantages indicate that the proposed methodology, especially MC bootstrapping samples, can be easily applied in Hill fitting and it also can be extended to other problems incorporating bootstrapping samples in general.

Nevertheless, despite the promising results from the proposed methodology especially for bootstrapping, one can observe that this method has one prominent limitation. During the generation of blocking data by adding Gaussian scatters, I assumed that the scatters follow Gaussian distribution. In Hill fitting, the Gaussian distribution of scatter data produces quite consistent results especially with sufficient dose-response data compared to MCMC. However, for other types of problems, the errors may follow other of distribution, thus one may need to change with different distribution sampling during MC simulation.

# [References]

1.	Hill, A.V., The possible effects of the aggregation of the molecules of hæmoglobin on its dissociation curves. The Journal of Physiology, 1910. 40: p. i--vii.

2.	Gesztelyi, R., et al., The Hill equation and the origin of quantitative pharmacology. Archive for History of Exact Sciences, 2012. 66(4): p. 427-438.

3.	Langmuir, I., THE ADSORPTION OF GASES ON PLANE SURFACES OF GLASS, MICA AND PLATINUM. Journal of the American Chemical Society, 1918. 40(9): p. 1361-1403.

4.	Neubig, R.R., et al., International Union of Pharmacology Committee on Receptor Nomenclature and Drug Classification. XXXVIII. Update on terms and symbols in quantitative pharmacology. Pharmacol Rev, 2003. 55(4): p. 597-606.

5.	Kenakin, T., Data-driven analysis in drug discovery. J Recept Signal Transduct Res, 2006. 26(4): p. 299-327.

6.	Motulsky, H. and A. Christopoulos. Fitting Models to Biological Data Using Linear and Nonlinear Regression: A Practical Guide to Curve Fitting. 2004.

7.	Black, J.W. and P. Leff, Operational Models of Pharmacological Agonism. Proceedings of the Royal Society of London. Series B, Biological Sciences, 1983. 220(1219): p. 141-162.

8.	Keller, F., et al., PK-PD curve-fitting problems with the Hill equation? Try one of the 1-exp functions derived from Hodgkin, Douglas or Gompertz. Int J Clin Pharmacol Ther, 2002. 40(1): p. 23-9.

9.	Giraldo, J., et al., Assessing the (a)symmetry of concentration-effect curves: empirical versus mechanistic models. Pharmacol Ther, 2002. 95(1): p. 21-45.

10.	Van der Graaf, P.H. and W.B. Stam, Analysis of receptor inactivation experiments with the operational model of agonism yields correlated estimates of agonist affinity and efficacy. Journal of Pharmacological and Toxicological Methods, 1999. 41(2): p. 117-125.

11.	Gintant, G.A., Preclinical Torsades-de-Pointes screens: advantages and limitations of surrogate and direct approaches in evaluating proarrhythmic risk. Pharmacol Ther, 2008. 119(2): p. 199-209.

12.	Sager, P.T., et al., Rechanneling the cardiac proarrhythmia safety paradigm: a meeting report from the Cardiac Safety Research Consortium. Am Heart J, 2014. 167(3): p. 292-300.

13.	Vicente, J., et al., Mechanistic Model-Informed Proarrhythmic Risk Assessment of Drugs: Review of the "CiPA" Initiative and Design of a Prospective Clinical Validation Study. Clin Pharmacol Ther, 2018. 103(1): p. 54-66.

14.	Strauss, D.G., et al., Comprehensive In Vitro Proarrhythmia Assay (CiPA) Update from a Cardiac Safety Research Consortium / Health and Environmental Sciences Institute / FDA Meeting. Ther Innov Regul Sci, 2019. 53(4): p. 519-525.

15.	Chang, K.C., et al., Uncertainty Quantification Reveals the Importance of Data Variability and Experimental Design Considerations for in Silico Proarrhythmia Risk Assessment. Front Physiol, 2017. 8: p. 917.

16.	Haario, H., et al., DRAM: Efficient adaptive MCMC. Statistics and Computing, 2006. 16(4): p. 339-354.

17.	Laine, M. and J. Tamminen, Aerosol model selection and uncertainty modelling by adaptive MCMC technique. Atmos. Chem. Phys., 2008. 8(24): p. 7697-7707.

18.	Soetaert, K. and T. Petzoldt, Inverse Modelling, Sensitivity and Monte Carlo Analysis in R Using Package FME. Journal of statistical software, 2010. 33: p. 1.

19.	Crumb, W.J., Jr., et al., An evaluation of 30 clinical drugs against the comprehensive in vitro proarrhythmia assay (CiPA) proposed ion channel panel. J Pharmacol Toxicol Methods, 2016. 81: p. 251-62.

20.	Levenberg, K., A METHOD FOR THE SOLUTION OF CERTAIN NON-LINEAR PROBLEMS IN LEAST SQUARES. Quarterly of Applied Mathematics, 1944. 2(2): p. 164-168.

21.	Marquardt, D.W., An Algorithm for Least-Squares Estimation of Nonlinear Parameters. Journal of the Society for Industrial and Applied Mathematics, 1963. 11(2): p. 431-441.

22.	Li, Z., et al., Improving the In Silico Assessment of Proarrhythmia Risk by Combining hERG (Human Ether-a-go-go-Related Gene) Channel-Drug Binding Kinetics and Multichannel Pharmacology. Circ Arrhythm Electrophysiol, 2017. 10(2): p. e004628.

# Appendix

## Code for Data processing

Prior to fitting procedure, there will be data processing steps that read experimental data, store it into arrays, sort it with respect to channel name, and average blocking data on each channel. These steps are described by several C++ functions and steps as follow:

- Read experimental data and find number of data per channel

int readDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr){

int iter;

iter = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && dataStr[i][2]==channelName){

iter = iter + 1;

}

}

return iter;

}

This function will return an integer value of the number of data on each channel of the drug.

- Store the data per channel into array

void storeDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr,double **dataPerChannel, double **data){

int iter;

iter = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && dataStr[i][2]==channelName){//change channelList index for other channel

dataPerChannel[iter][0] = data[i][0];//conc

dataPerChannel[iter][1] = data[i][1];//block

iter = iter + 1;

}

}

return;

}

The output of this function is the dataPerChannel array that contains all experimental data per channel.

- Reading data per channel to obtain number of data having same amount of drug concentration

int readConcPerChannel(int numberOfDrugDataPerChannel, double **dataPerChannel){

int iter;

double conc;

iter = 0;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

if(i==0){

conc = dataPerChannel[i][0];//conc variable

iter = iter + 1;

}

else{

if(dataPerChannel[i][0]!=conc){

conc = dataPerChannel[i][0];//conc variable

iter = iter + 1;

}

}

}

return iter;

}

It will return an integer value of the number of different concentrations within the data per channel.

- Calculating the averaged data for each drug concentration as described in [Figure 2. 2]

void findAveragedData(int numberOfDrugDataPerChannel, double **dataPerChannel, double **sortedDataPerChannel){

int iter,iter2;

double block,conc;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

if(i==0){//first data of dataPerChannel

conc = dataPerChannel[i][0];//conc variable

block = dataPerChannel[i][1];//block variable

iter = 1;//counting number of same conc values

iter2 = 0;//row of sortedDataPerChannel

}

else{

if(dataPerChannel[i][0]==conc){

if(i<numberOfDrugDataPerChannel-1){

block = block + dataPerChannel[i][1];//adding block variables

iter = iter + 1;

}

else{//last data of dataPerChannel

block = block + dataPerChannel[i][1];//adding block variables

iter = iter + 1;

block = block/iter;

sortedDataPerChannel[iter2][0] = conc;

sortedDataPerChannel[iter2][1] = block;

//std::cout << i << " Sorted " << iter2 << " has value of conc = " << conc;

//std::cout << " and average block " << block << std::endl;

}

}

else{

block = block/iter;//average block

sortedDataPerChannel[iter2][0] = conc;

sortedDataPerChannel[iter2][1] = block;

//std::cout << i << " Sorted " << iter2 << " has value of conc = " << conc;

//std::cout << " and average block " << block << std::endl;

conc = dataPerChannel[i][0];//set new conc variable

block = dataPerChannel[i][1];//set new block variable

iter = 1;//restart counting same conc values

iter2 = iter2 + 1;//next row of sortedDataPerChannel

}

}

}

return;

}

The sorted data will be the stored in sortedDataPerChannel array.

## Code for linear and nonlinear LSM

As stated in Section 2.1.1, especially (Eq. 2.1.2) to (Eq. 2.1.4) that express the linear equation for linear LSM fitting, the implementation of LSM relies on ALGLIB C++where the main calculation is as follow:

- The minimum requirement to conduct the linear LSM fitting is declared first.

The algorithm requires that the experimental data have at least 4 different drug concentrations for each channel. It is implemented as:

if(numberOfDataLinFit<4){

std::cout << "Too few number of distinct conc values to fit Hill equation " << std::endl;

continue;

}

- If the minimum requirement is fulfilled, the next step is to calculate the actual initial guess of logIC50 and h. If the data is either dominated by very low or very high blocking, the logIC50 and h will be set as follow:

//criteria for linear fitting

if(flag1==numberOfDataLinFit){//all of the sortedDataPerChannell are very small blocking

//c0 = -log10(gsl_vector_max(dataConc));//log10(IC50)

c0 = -log10(*std::max_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1;//Hill coefficient

}

else if(flag2==numberOfDataLinFit){//all of the sortedDataPerChannell are very big blocking

c0 = -log10(*std::min_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1.0;//Hill coefficient

}

- If neither very low nor very high blocking dominate the data or ,in other words, we have a regular data, we can securely conduct linear LSM as follow:

//ignore data with dataFlag!=0

numberOfDataLinFit = numberOfDataLinFit - flag1 - flag2;

//std::cout << "This is the number of data to fit: " << numberOfDataLinFit << std::endl;

if(2 <= numberOfDataLinFit){

dataX = new double [numberOfDataLinFit];

dataY = new double [numberOfDataLinFit];

ignoreSomeData(numberOfDataLinFit,flag1,flag2,dataFlag,dataX,dataY,sortedDataPerChannel);

//initialize linear lsm environment

real_1d_array ylin;

real_2d_array fmatrix;

ylin.setlength(numberOfDataLinFit);

fmatrix.setlength(numberOfDataLinFit,2);

for(int i=0;i<numberOfDataLinFit;i++){

ylin[i] = dataY[i];

fmatrix[i][0] = 1;

fmatrix[i][1] = dataX[i];

}

int_t infolin;

real_1d_array clin;

lsfitreport replin;

// Fitting without weights

lsfitlinear(ylin, fmatrix, infolin, clin, replin);

//printf("# best fit: Y = %g + %g X\n",clin[0],clin[1]);

//printf("# h = %g, log10IC50 = %g, IC50 = %g\n",clin[1],-clin[0]/clin[1],pow(10.0,-clin[0]/clin[1]));

c0 = clin[0];

c1 = clin[1];

delete [] dataX;

delete [] dataY;

Once the linear LSM finish, the initial logIC50 and h are further processed into nonlinear LSM fitting by following the procedure explained in Section 2.1.2. The implementation of nonlinear LSM fitting is done by using the following function:

void nonLinearFit(int numberOfDrugDataPerChannel, double c0, double c1, double *log10IC50, double *h, double **dataPerChannel){

double conc;

real_2d_array x;

real_1d_array y;

x.setlength(numberOfDrugDataPerChannel,1);

y.setlength(numberOfDrugDataPerChannel);

for(int i=0;i<numberOfDrugDataPerChannel;i++){

x[i][0] = dataPerChannel[i][0];//drug concentration

y[i] = dataPerChannel[i][1];//blocking percentage

}

real_1d_array c;//parameters to fit

c.setlength(2);

//check the linear fitting results

//lower and upper bounds for h

if(c1 <= 0 || 10 <= c1){

//set new h parameter

c1 = 0.9;

}

//lower and upper bounds for log10IC50

if(-c0/c1 <= -10 || 10 <= -c0/c1){

//find average conc value

conc = 0.0;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

conc = conc + dataPerChannel[i][0];//conc

}

conc = conc/numberOfDrugDataPerChannel;

//new log10IC50 = -c0/c1

c0 = -log10(conc)*c1;

}

c[0] = -c0/c1;//log10IC50

c[1] = c1;//h

real_1d_array bndl;

bndl.setlength(2);

bndl[0] = -10.0;

bndl[1] = 0;

real_1d_array bndu;

bndu.setlength(2);

bndu[0] = 10.0;

bndu[1] = 10.0;

double epsx = 0.000001;

ae_int_t maxits = 0;

ae_int_t info;

lsfitstate state;

lsfitreport rep;

double diffstep = 0.0001;

lsfitcreatef(x, y, c, diffstep, state);

lsfitsetbc(state, bndl, bndu);

lsfitsetcond(state, epsx, maxits);

alglib::lsfitfit(state, function_cx_1_func);

lsfitresults(state, info, c, rep);

//store results

*log10IC50 = c[0];

*h = c[1];

return;

}

This function will result the optimum log10IC50 and h.

## Code for Monte Carlo simulation

The implementation of Monte Carlo simulation is basically the same as the previous Appendix B except that it repeats the LSM fitting as many bootstrap samples as we need and it has different set of data on every iteration. There are two available schemes as in [Figure 2. 1]. In the first scheme, the data is generated by using optimal or best-fit parameters from LSM fitting with Gaussian noise. The implementation of data generator for this scheme is as follow:

for(int j=0;j<numberOfDrugDataPerChannel;j++){

conc = dataPerChannel[j][0];//data of drug concentration

//block = dataPerChannel[j][1];//obtaining blocking from actual data

block = blocking(log10IC50_Best,conc,h_Best);//obtaining blocking from previous best-fit value

noise = gsl_ran_gaussian(r,SD);

dataPerChannelRand[j][0] = conc;//conc

if(block+noise<0){

dataPerChannelRand[j][1] = 0;//block

}

else{

dataPerChannelRand[j][1] = block+noise;//block

}

//printf("Conc: %f Blocking: %f Noise: %f \n",conc,block,noise);

}

The noise = gsl_ran_gaussian(r,SD) is given to the blocking data in every iteration. Each iteration will result different noise thus generates a different set of data. Meanwhile, the second scheme generates data from actual experimental data with Gaussian noise simply by changing the following expression:

block = blocking(log10IC50_Best,conc,h_Best);//obtaining blocking from previous best-fit value

to

block = dataPerChannel[j][1];//obtaining blocking from actual data

## Complete code for Hill fitting with LSM and Monte Carlo simulation

#include "stdafx.h"

#include <stdlib.h>

#include <stdio.h>

#include <math.h>

#include "interpolation.h"

#include <iostream>

#include <iomanip>

#include <fstream>

#include <sstream>

#include <cstring>

#include <string>

#include <algorithm>

#include <math.h>

#include <gsl/gsl_rng.h>

#include <gsl/gsl_randist.h>

using namespace alglib;

//function for linear fitting

double Y(double block){

return log10(1.0/(1.0-block/100.0)-1.0);

}

double X(double conc){

return log10(conc);

}

double blocking(double log10IC50, double conc, double h){

return 100.0*(1.0-1.0/(1.0+pow((conc/pow(10.0,log10IC50)),h)));

}

void function_cx_1_func(const real_1d_array &c, const real_1d_array &x, double &func, void *ptr);

void readData(std::string fileName, double **data, std::string **dataStr);

int readChannels(int numberOfData, std::string drugName, std::string** dataStr);

int readDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr);

void storeDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr,double **dataPerChannel, double **data);

int readConcPerChannel(int numberOfDrugDataPerChannel, double **dataPerChannel);

void storeChannelList(int numberOfData, std::string drugName, std::string** dataStr, std::string* channelList);

void findAveragedData(int numberOfDrugDataPerChannel, double **dataPerChannel, double **sortedDataPerChannel);

void ignoreSomeData(int numberOfDataLinFit, int flag1, int flag2, int* dataFlag, double* dataX, double* dataY, double** sortedDataPerChannel);

void nonLinearFit(int numberOfDrugDataPerChannel, double c0, double c1, double *log10IC50, double *h, double **dataPerChannel);

double stdev(int numberOfDrugDataPerChannel, double log10IC50, double h, double ** dataPerChannel){

double result = 0.0, block, conc;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

conc = dataPerChannel[i][0];

block = dataPerChannel[i][1];

result = result + pow(blocking(log10IC50,conc,h)-block,2.0);

}

result = sqrt(result/(numberOfDrugDataPerChannel-2));

return result;

}

int main(int argc, char **argv)

{

//

// In this example we demonstrate Hill fitting by

// f(c,x)=100*(1-1/(1+pow(x/pow(10,c0),c1)))

// subject to bound constraints

//     -10.0 <= c0 <= 10.0

//       0.0 <= c1 <= 10.0

// using function value only.

// c0 = log10IC50 and c1 = h.

//

// Gradient is estimated using combination of numerical differences

// and secant updates. diffstep variable stores differentiation step

// (we have to tell algorithm what step to use).

//

//declaration for linear fitting

int numberOfData,numberOfChannels,numberOfDrugDataPerChannel;

int *dataFlag,numberOfDataLinFit;

double conc,block,noise,eps=0.01;

double c0,c1,cov00,cov01,cov11,chisq;

double *weight,SD,log10IC50,h;

double **dataRaw,**dataPerChannel,**dataPerChannelRand,**sortedDataPerChannel,**finalSamples;

std::string **dataStr,*channelList,tempString,line,fileName,oFileName,bFileName,sFileName,headerSFile;

std::string drugName,channelName,drug,channel,units,pacing;

//initialization

//==============

if (argc != 2){

fprintf (stderr,"usage: ./lsm.out drugName\n");

exit (-1);

}

drugName = argv[1];

bFileName = "data/bootstrap_"+drugName+".csv";

sFileName = "data/IC50_samples_"+drugName+".csv";

//open output file to store results

std::ofstream bFile(bFileName);

std::ofstream sFile(sFileName);

bFile << "channel,log10IC50,h" <<  std::endl;

//storing data from drug_block.csv

//================================

fileName = "data/drug_block_validation.csv";

std::ifstream inputFile(fileName);

if(!inputFile.is_open()){

std::cout << "Input drug_block.csv file failed to open" << std::endl;

std::cout << "File name:" << fileName << std::endl;

std::cout << "==============================" << std::endl;

}

//skip file header

getline(inputFile,line);

//calculate number of drug data

numberOfData = 0;

while(getline(inputFile,line)){

numberOfData = numberOfData + 1;

}

inputFile.clear();

inputFile.seekg(0, inputFile.beg);

//preparing data array for conc and block

dataRaw = new double * [numberOfData];

for(int i=0;i<numberOfData;i++){

dataRaw[i] = new double [2];

}

//preparing an array for drug,channel,units and pacing

dataStr = new std::string* [numberOfData];

for(int i=0;i<numberOfData;i++){

dataStr[i] = new std::string [4];

}

//store inputFile into data and dataStr matrix

readData(fileName,dataRaw,dataStr);

//collecting name of channels

numberOfChannels = readChannels(numberOfData,drugName,dataStr);

//preparing an array for channel list

channelList = new std::string [numberOfChannels];

//storing channel names into channelList

storeChannelList(numberOfData,drugName,dataStr,channelList);

//preparing to store the finalSamples for sFile (drug IC50 bootstrap samples)

finalSamples = new double *[2000];

for(int i=0;i<2000;i++){

finalSamples[i] = new double [2*numberOfChannels];

}

//Start calculation value for all channels

//========================================

for(int channelNum=0;channelNum<numberOfChannels;channelNum++){

//collecting drug data per channel

channelName = channelList[channelNum];

headerSFile = headerSFile+channelName+"_IC50,"+channelName+"_h,";//Header for IC50 2,000 samples file

numberOfDrugDataPerChannel = readDataPerChannel(numberOfData,drugName,channelName,dataStr);

//storing dataPerChannel

dataPerChannel = new double * [numberOfDrugDataPerChannel];

dataPerChannelRand = new double * [numberOfDrugDataPerChannel];//For storing dataPerChannel with random noise

for(int i=0;i<numberOfDrugDataPerChannel;i++){

dataPerChannel[i] = new double [2];

dataPerChannelRand[i] = new double [2];

}

storeDataPerChannel(numberOfData,drugName,channelName,dataStr,dataPerChannel,dataRaw);

//find number of different conc values

numberOfDataLinFit = readConcPerChannel(numberOfDrugDataPerChannel,dataPerChannel);

std::cout << "Drug " << drugName << " with";

std::cout << " channel "<< channelName << " has ";

std::cout << numberOfDrugDataPerChannel << " number of data per channel";

std::cout << " and " << numberOfDataLinFit << " number of different conc values" << std::endl;

sortedDataPerChannel = new double * [numberOfDataLinFit];

for(int i=0;i<numberOfDataLinFit;i++){

sortedDataPerChannel[i] = new double [2];

}

//finding average for sortedDataPerChannel

findAveragedData(numberOfDrugDataPerChannel,dataPerChannel,sortedDataPerChannel);

//linear fitting procedure

//========================

int flag1,flag2;

double *dataX,*dataY,*dataConc;

//store initial dataFlag

dataFlag = new int [numberOfDataLinFit];

for(int i=0;i<numberOfDataLinFit;i++){

dataFlag[i] = 0;

}

//check the total number of sortedDataPerChannel

if(numberOfDataLinFit<4){

std::cout << "Too few number of distinct conc values to fit Hill equation " << std::endl;

continue;

}

else{

//counting flag1 for too many very small blocking

std::cout << "Count many very small blocking " << std::endl;

flag1 = 0;

for(int i=0;i<numberOfDataLinFit;i++){

if(sortedDataPerChannel[i][1]<eps){

flag1 = flag1 + 1;

dataFlag[i] = 1;

}

}

std::cout << "Number of too small blocking: " << flag2 << std::endl;

//counting flag2 for too many very big blocking

std::cout << "Count many very big blocking " << std::endl;

flag2 = 0;

for(int i=0;i<numberOfDataLinFit;i++){

if(sortedDataPerChannel[i][1]>100.0-eps){

flag2 = flag2 + 1;

dataFlag[i] = 2;

}

}

std::cout << "Number of too big blocking: " << flag2 << std::endl;

//copying conc data from sortedDataPerChannel

std::cout << "check" << std::endl;

dataConc = new double [numberOfDataLinFit];

for(int i=0;i<numberOfDataLinFit;i++){

dataConc[i] = sortedDataPerChannel[i][1];

}

//criteria for linear fitting

if(flag1==numberOfDataLinFit){//all of the sortedDataPerChannell are very small blocking

c0 = -log10(*std::max_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1;//Hill coefficient

std::cout << "check2" << std::endl;

}

else if(flag2==numberOfDataLinFit){//all of the sortedDataPerChannell are very big blocking

c0 = -log10(*std::min_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1.0;//Hill coefficient

std::cout << "check3" << std::endl;

}

else{

//ignore data with dataFlag!=0

numberOfDataLinFit = numberOfDataLinFit - flag1 - flag2;

std::cout << "This is the number of data to fit: " << numberOfDataLinFit << std::endl;

if(2 <= numberOfDataLinFit){

dataX = new double [numberOfDataLinFit];

dataY = new double [numberOfDataLinFit];

ignoreSomeData(numberOfDataLinFit,flag1,flag2,dataFlag,dataX,dataY,sortedDataPerChannel);

//initialize linear lsm environment

real_1d_array ylin;

real_2d_array fmatrix;

ylin.setlength(numberOfDataLinFit);

fmatrix.setlength(numberOfDataLinFit,2);

for(int i=0;i<numberOfDataLinFit;i++){

ylin[i] = dataY[i];

fmatrix[i][0] = 1;

fmatrix[i][1] = dataX[i];

}

ae_int_t infolin;

real_1d_array clin;

lsfitreport replin;

// Fitting without weights

lsfitlinear(ylin, fmatrix, infolin, clin, replin);

printf("# best fit: Y = %g + %g X\n",clin[0],clin[1]);

printf("# h = %g, log10IC50 = %g, IC50 = %g\n",clin[1],-clin[0]/clin[1],pow(10.0,-clin[0]/clin[1]));

c0 = clin[0];

c1 = clin[1];

delete [] dataX;

delete [] dataY;

}

else{

std::cout << "check4" << std::endl;

c1 = 0.9;//Hill coeff

//log10IC50 = -log10(conc)*h

c0 = -log10(0.5*(*std::max_element(dataConc,dataConc+numberOfDataLinFit)-*std::min_element(dataConc,dataConc+numberOfDataLinFit)))*c1;

}

}

delete [] dataConc;

}//else

//Nonlinear lsm procedure

//=======================

double log10IC50_Best, h_Best,SD;

nonLinearFit(numberOfDrugDataPerChannel,c0,c1,&log10IC50_Best,&h_Best,dataPerChannel);

SD = stdev(numberOfDrugDataPerChannel,log10IC50_Best,h_Best,dataPerChannel);

printf("Best fit parameters from LSM:\n");

printf("log10IC50 = %f, h = %f \n",log10IC50_Best,h_Best);

printf("SD of residuals = %f\n",SD);

printf("=============================\n");

//Start Monte Carlo simulation to generate bootstrap

printf("Starting Monte Carlo simulation ...\n");

double *MClog10IC50Samples,*MChSamples;

MClog10IC50Samples = new double [2000];

MChSamples = new double [2000];

for(int MC=0;MC<2000;MC++){

//Modifying the block percentage on dataPerChannelRand with Gaussian noise

const gsl_rng_type * M;

gsl_rng * r;

gsl_rng_env_setup();

M = gsl_rng_default;

r = gsl_rng_alloc(M);

gsl_rng_set(r,clock());

for(int j=0;j<numberOfDrugDataPerChannel;j++){

conc = dataPerChannel[j][0];//data of drug concentration

//block = dataPerChannel[j][1];//obtaining blocking from actual data

block = blocking(log10IC50_Best,conc,h_Best);//obtaining blocking from previous best-fit value

noise = gsl_ran_gaussian(r,SD);

dataPerChannelRand[j][0] = conc;//conc

if(block+noise<0){

dataPerChannelRand[j][1] = 0;//block

}

else{

dataPerChannelRand[j][1] = block+noise;//block

}

//printf("Conc: %f Blocking: %f Noise: %f \n",conc,block,noise);

}

//find number of different conc values

numberOfDataLinFit = readConcPerChannel(numberOfDrugDataPerChannel,dataPerChannelRand);

//finding average for sortedDataPerChannel

findAveragedData(numberOfDrugDataPerChannel,dataPerChannelRand,sortedDataPerChannel);

//linear fitting procedure Monte Carlo

//====================================

int flag1,flag2;

double *dataX,*dataY,*dataConc;

//store initial dataFlag

dataFlag = new int [numberOfDataLinFit];

for(int i=0;i<numberOfDataLinFit;i++){

dataFlag[i] = 0;

}

//check the total number of sortedDataPerChannel

if(numberOfDataLinFit<4){

std::cout << "Too few number of distinct conc values to fit Hill equation " << std::endl;

continue;

}

else{

//counting flag1 for too many very small blocking

flag1 = 0;

for(int i=0;i<numberOfDataLinFit;i++){

if(sortedDataPerChannel[i][1]<eps){

flag1 = flag1 + 1;

dataFlag[i] = 1;

}

}

//counting flag2 for too many very big blocking

flag2 = 0;

for(int i=0;i<numberOfDataLinFit;i++){

if(sortedDataPerChannel[i][1]>100.0-eps){

flag2 = flag2 + 1;

dataFlag[i] = 2;

}

}

//copying conc data from sortedDataPerChannel

dataConc = new double [numberOfDataLinFit];

for(int i=0;i<numberOfDataLinFit;i++){

dataConc[i] = sortedDataPerChannel[i][1];

}

//criteria for linear fitting

if(flag1==numberOfDataLinFit){//all of the sortedDataPerChannell are very small blocking

c0 = -log10(*std::max_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1;//Hill coefficient

}

else if(flag2==numberOfDataLinFit){//all of the sortedDataPerChannell are very big blocking

c0 = -log10(*std::min_element(dataConc,dataConc+numberOfDataLinFit));//log10(IC50)

c1 = 1.0;//Hill coefficient

}

else{

//ignore data with dataFlag!=0

numberOfDataLinFit = numberOfDataLinFit - flag1 - flag2;

if(2 <= numberOfDataLinFit){

dataX = new double [numberOfDataLinFit];

dataY = new double [numberOfDataLinFit];

ignoreSomeData(numberOfDataLinFit,flag1,flag2,dataFlag,dataX,dataY,sortedDataPerChannel);

//initialize linear lsm environment

real_1d_array ylin;

real_2d_array fmatrix;

ylin.setlength(numberOfDataLinFit);

fmatrix.setlength(numberOfDataLinFit,2);

for(int i=0;i<numberOfDataLinFit;i++){

ylin[i] = dataY[i];

fmatrix[i][0] = 1;

fmatrix[i][1] = dataX[i];

}

ae_int_t infolin;

real_1d_array clin;

lsfitreport replin;

// Fitting without weights

lsfitlinear(ylin, fmatrix, infolin, clin, replin);

c0 = clin[0];

c1 = clin[1];

delete [] dataX;

delete [] dataY;

}

else{

c1 = 0.9;//Hill coeff

//log10IC50 = -log10(conc)*h

c0 = -log10(0.5*(*std::max_element(dataConc,dataConc+numberOfDataLinFit)-*std::min_element(dataConc,dataConc+numberOfDataLinFit)))*c1;

}

}

delete [] dataConc;

}//else

//nonLinear fitting procedure Monte Carlo

//=======================================

double log10IC50,h;

nonLinearFit(numberOfDrugDataPerChannel,c0,c1,&log10IC50,&h,dataPerChannelRand);

bFile << channelName << "," << log10IC50 << "," << h << std::endl;

finalSamples[MC][channelNum*2] = pow(10,log10IC50);

finalSamples[MC][channelNum*2+1] = h;

MClog10IC50Samples[MC] = log10IC50;

MChSamples[MC] = h;

gsl_rng_free(r);

}

//Sort the MC samples to obtain uncertainties (95% confidence intervals)

std::sort(MClog10IC50Samples,MClog10IC50Samples+2000);

std::cout << "Upper bound of 95% CI interval for log10IC50: " << MClog10IC50Samples[1899] << std::endl;

std::cout << "Lower bound of 95% CI interval for log10IC50: " << MClog10IC50Samples[99] << std::endl;

std::sort(MChSamples,MChSamples+2000);

std::cout << "Upper bound of 95% CI interval for h: " << MChSamples[1899] << std::endl;

std::cout << "Lower bound of 95% CI interval for h: " << MChSamples[99] << std::endl;

//Finish Monte Carlo simulation to generate bootstrap

printf("Monte Carlo simulation is finished...\n");

printf("=====================================\n");

delete [] MClog10IC50Samples;

delete [] MChSamples;

delete [] dataPerChannelRand;

delete [] dataPerChannel;

delete [] sortedDataPerChannel;

}//channel

//print sFile

sFile << headerSFile << std::endl;

for(int i=0;i<2000;i++){

for(int j=0;j<numberOfChannels;j++){

sFile << finalSamples[i][j*2] << "," << finalSamples[i][j*2+1] << ",";

}

sFile << std::endl;

}

delete [] finalSamples;

delete [] dataRaw;

delete [] dataStr;

delete [] channelList;

sFile.close();

bFile.close();

inputFile.close();

//outputFile.close();

return 0;

}

void function_cx_1_func(const real_1d_array &c, const real_1d_array &x, double &func, void *ptr)

{

// this callback calculates f(c,x)=100*(1-1/(1+pow(x/pow(10,c0),c1)))

// where x is a drug concentration on X-axis and c is adjustable parameter

// c0 = log10IC50, c1 = h

func = 100*(1-1/(1+pow(x[0]/pow(10,c[0]),c[1])));

}

void readData(std::string fileName, double **data, std::string **dataStr){

int iter;

double block,conc;

std::string line,tempString,drug,channel,units,pacing;

std::ifstream inputFile(fileName);

//skip file header

getline(inputFile,line);

iter = 0;

while(getline(inputFile,line)){

std::stringstream ss(line);

getline(ss,tempString,',');//drug

drug = tempString;

getline(ss,tempString,',');//conc

conc = atof(tempString.c_str());

getline(ss,tempString,',');//units

units = tempString;

getline(ss,tempString,',');//channel

channel = tempString;

getline(ss,tempString,',');//block

block = atof(tempString.c_str());

getline(ss,tempString,',');//pacing

pacing = tempString;

//store string and numerical data into arrays

data[iter][0] = conc;//store conc data

data[iter][1] = block;//store block data

dataStr[iter][0] = drug;//store dataStr

dataStr[iter][1] = units;//store dataStr

dataStr[iter][2] = channel;//store dataStr

dataStr[iter][3] = pacing;//store dataStr

iter = iter + 1;

}

inputFile.close();

return;

}

int readChannels(int numberOfData, std::string drugName, std::string** dataStr){

int numberOfChannels;

std::string channelName;

numberOfChannels = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && numberOfChannels==0){

channelName = dataStr[i][2];//first channel

numberOfChannels = numberOfChannels + 1;

}

else if (dataStr[i][0]==drugName && numberOfChannels>0){

if(dataStr[i][2]!=channelName){

channelName = dataStr[i][2];//next channel

numberOfChannels = numberOfChannels + 1;

}

}

}

return numberOfChannels;

}

int readDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr){

int iter;

iter = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && dataStr[i][2]==channelName){

iter = iter + 1;

}

}

return iter;

}

void storeDataPerChannel(int numberOfData, std::string drugName, std::string channelName, std::string** dataStr,double **dataPerChannel, double **data){

int iter;

iter = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && dataStr[i][2]==channelName){//change channelList index for other channel

dataPerChannel[iter][0] = data[i][0];//conc

dataPerChannel[iter][1] = data[i][1];//block

iter = iter + 1;

}

}

return;

}

int readConcPerChannel(int numberOfDrugDataPerChannel, double **dataPerChannel){

int iter;

double conc;

iter = 0;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

if(i==0){

conc = dataPerChannel[i][0];//conc variable

iter = iter + 1;

}

else{

if(dataPerChannel[i][0]!=conc){

conc = dataPerChannel[i][0];//conc variable

iter = iter + 1;

}

}

}

return iter;

}

void storeChannelList(int numberOfData, std::string drugName, std::string** dataStr, std::string* channelList){

int iter;

std::string channelName;

iter = 0;

for(int i=0;i<numberOfData;i++){

if(dataStr[i][0]==drugName && iter==0){

channelName = dataStr[i][2];//first channel

channelList[iter] = channelName;

iter = iter + 1;

}

else if (dataStr[i][0]==drugName && iter>0){

if(dataStr[i][2]!=channelName){

channelName = dataStr[i][2];//next channel

channelList[iter] = channelName;

iter = iter + 1;

}

}

}

return;

}

void findAveragedData(int numberOfDrugDataPerChannel, double **dataPerChannel, double **sortedDataPerChannel){

int iter,iter2;

double block,conc;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

if(i==0){//first data of dataPerChannel

conc = dataPerChannel[i][0];//conc variable

block = dataPerChannel[i][1];//block variable

iter = 1;//counting number of same conc values

iter2 = 0;//row of sortedDataPerChannel

}

else{

if(dataPerChannel[i][0]==conc){

if(i<numberOfDrugDataPerChannel-1){

block = block + dataPerChannel[i][1];//adding block variables

iter = iter + 1;

}

else{//last data of dataPerChannel

block = block + dataPerChannel[i][1];//adding block variables

iter = iter + 1;

block = block/iter;

sortedDataPerChannel[iter2][0] = conc;

sortedDataPerChannel[iter2][1] = block;

}

}

else{

block = block/iter;//average block

sortedDataPerChannel[iter2][0] = conc;

sortedDataPerChannel[iter2][1] = block;

conc = dataPerChannel[i][0];//set new conc varibale

block = dataPerChannel[i][1];//set new block variable

iter = 1;//restart counting same conc values

iter2 = iter2 + 1;//next row of sortedDataPerChannel

}

}

}

return;

}

void ignoreSomeData(int numberOfDataLinFit, int flag1, int flag2, int* dataFlag, double* dataX, double* dataY, double** sortedDataPerChannel){

int iter;

iter = 0;

for(int i=0; i<numberOfDataLinFit+flag1+flag2;i++){

if(dataFlag[i]==0){

dataX[iter] = X(sortedDataPerChannel[i][0]);//conc

dataY[iter] = Y(sortedDataPerChannel[i][1]);//block

iter = iter + 1;

}

}

return;

}

void nonLinearFit(int numberOfDrugDataPerChannel, double c0, double c1, double *log10IC50, double *h, double **dataPerChannel){

double conc;

real_2d_array x;

real_1d_array y;

x.setlength(numberOfDrugDataPerChannel,1);

y.setlength(numberOfDrugDataPerChannel);

for(int i=0;i<numberOfDrugDataPerChannel;i++){

x[i][0] = dataPerChannel[i][0];//drug concentration

y[i] = dataPerChannel[i][1];//blocking percentage

}

real_1d_array c;//parameters to fit

c.setlength(2);

//check the linear fitting results

//lower and upper bounds for h

if(c1 <= 0 || 10 <= c1){

//set new h parameter

c1 = 0.9;

}

//lower and upper bounds for log10IC50

if(-c0/c1 <= -10 || 10 <= -c0/c1){

//find average conc value

conc = 0.0;

for(int i=0;i<numberOfDrugDataPerChannel;i++){

conc = conc + dataPerChannel[i][0];//conc

}

conc = conc/numberOfDrugDataPerChannel;

//new log10IC50 = -c0/c1

c0 = -log10(conc)*c1;

}

c[0] = -c0/c1;//log10IC50

c[1] = c1;//h

real_1d_array bndl;

bndl.setlength(2);

bndl[0] = -10.0;

bndl[1] = 0;

real_1d_array bndu;

bndu.setlength(2);

bndu[0] = 10.0;

bndu[1] = 10.0;

double epsx = 0.000001;

ae_int_t maxits = 0;

ae_int_t info;

lsfitstate state;

lsfitreport rep;

double diffstep = 0.0001;

lsfitcreatef(x, y, c, diffstep, state);

lsfitsetbc(state, bndl, bndu);

lsfitsetcond(state, epsx, maxits);

alglib::lsfitfit(state, function_cx_1_func);

lsfitresults(state, info, c, rep);

//store results

*log10IC50 = c[0];

*h = c[1];

return;

}

Uncertainty Quantification through the Monte Carlo method for Parameter Estimation of In silico Model used for Drug Toxicity Assessment

Thesis for Master of Engineering

December 2020

Ali Ikhsanul Qauli